Nomenclature

Selection of data files is based on specific nomenclature criteria

3 letter suffixes indicate the type of file

E00	-  ArcInfo Export File

TIF	-  TIFF file

TFW	-  TIFF registration file

Note:  When copying the TIFF file over to be used in an Arc software package the TFW file must travel with the TIF file.  Both must be copied to the same location.

Coal seams are abbreviated as shown in Coal codes.xls

A file named  Ba.e00 indicates that it is a coverage of the B Coal Seam (Lower Kittanning) that was developed from Mylar 1 of 2 for that quad.  A file Pittb.e00 indicates a file from the Pittsburgh coal seam developed from Mylar 2 of 2.  A file Ba.tif is a TIFF file from the B seam created from the first mylar of the B seam for that quad.

A file named bw indicates that this is the coverage created from the WPA mine maps.  In some instances the coverages ending in ‘w’ will also show the permits applications that were denied.

Quads are abbreviated as shown in distribution.xls

The Column Date Update in the CD-Code.xls spreadsheet is the date the quad is updated to.  All plots previous to that date have been added to the mylar overlay.

These coverages were developed from the Harrisburg Bureau of Mining and Reclamation mylar map system that was developed from various and multiple sources.  These coverages show the extent of permitting to the best of the draftsman’s ability.  No attempt was made to correlate coal seams when the quads were digitized.  Some coverages for coal seams may appear where the coal seam does not exist.  Care should be taken when these anomalies are discovered.  All reasonable attempts to correct the coverages will be made when sufficient data is provided to the Bureau of Mining and Reclamation.  These coverages are presented to give a reasonable guideline of what is on the ground at that area.  These coverages should not be sole means of decision-making and are in no way a substitute for actual on the ground observation.

Even though the term “Noncoal” has gone out of vogue it is used here as coverage names and image names because it will conform to the 8 dot 3 DOS naming convention.  Recognizing that many users of this data may not have operating systems that recognize names large than the 8 dot 3 system and finding no word better suited it was decided for this edition to utilize the term “noncoal”.